Step 5: Reliability Testing

Goal

Reliability testing checks whether a device works consistently over time and under repeated use.

It helps find:

- Random failures

- Overheating issues

- Memory leaks

- Connection drops

- Hardware instability

- Intermittent failures

Test 1: 100 Reboot Cycles

Objective

Verify the device can repeatedly reboot without errors.

Procedure

Cycle 1

Power ON Device
Wait for Boot Completion
Check Power Status
Check WiFi Connection
Check Camera Function
Record Result

Repeat

Reboot Device
Check Functions
Record Result

Repeat:

100 Times

Example Log

| Cycle | Power | WiFi | Camera | Result |
| --- | --- | --- | --- | --- |
| 1 | Pass | Pass | Pass | Pass |
| 2 | Pass | Pass | Pass | Pass |
| 3 | Pass | Fail | Pass | Fail |
| 4 | Pass | Pass | Pass | Pass |

Example Failure

Cycle 37
WiFi Not Detected

This indicates an intermittent failure.

Test 2: 24-Hour Run Test

Objective

Verify long-term stability.

Procedure

Start device:

08:00 AM

Run continuously:

24 Hours

Check every hour:

Power
Temperature
CPU Usage
Memory Usage
Network Connection

Example Monitoring Sheet

| Hour | Power | Network | Temp | Result |
| --- | --- | --- | --- | --- |
| 1 | Pass | Pass | 45°C | Pass |
| 2 | Pass | Pass | 47°C | Pass |
| 3 | Pass | Fail | 48°C | Fail |

Example Intermittent Failure

Network Disconnects
Only After 12 Hours

Functional testing may not find this issue.

Reliability testing will.

Test 3: Stress Testing

Objective

Push device to maximum load.

CPU Stress

Linux:

stress --cpu 4 --timeout 600

Run:

10 Minutes

Observe:

Temperature
Performance
Crashes

Memory Stress

stress --vm 2 --vm-bytes 1G --timeout 600

Network Stress

Transfer large files repeatedly.

10 GB Data Transfer

Check:

Packet Loss
Disconnects
Speed Drops

Document Results

Example CSV

Test_ID,Cycle,Power,WiFi,Camera,Result
T001,1,Pass,Pass,Pass,Pass
T001,2,Pass,Pass,Pass,Pass
T001,3,Pass,Fail,Pass,Fail

SQL Storage

CREATE TABLE reliability_test (
    Test_ID VARCHAR(20),
    Cycle_No INT,
    Power_Status VARCHAR(10),
    WiFi_Status VARCHAR(10),
    Camera_Status VARCHAR(10),
    Result VARCHAR(10)
);

Python Analysis

import pandas as pd

df = pd.read_csv("reliability_test.csv")

fails = df[df["Result"] == "Fail"]

print("Total Failures:", len(fails))
print(fails)

Finding Intermittent Failures

What is an Intermittent Failure?

A problem that:

Fails Sometimes
Passes Sometimes
Cannot Be Reproduced Every Time

Example:

| Cycle | WiFi |
| --- | --- |
| 1 | Pass |
| 2 | Pass |
| 3 | Fail |
| 4 | Pass |
| 5 | Pass |
| 6 | Fail |

This is intermittent.

SQL Query

Find WiFi failures:

SELECT *
FROM reliability_test
WHERE WiFi_Status='Fail';

Python Query

wifi_failures = df[df["WiFi_Status"] == "Fail"]

print(wifi_failures)